tinyCPU

Architecture Specification & Programming Reference

Version 1.2

# 1. Overview

tinyCPU is a simple, 8-bit educational processor designed to illustrate core concepts in computer architecture: instruction fetch, decode, and execute cycles; a general-purpose register file; a flat memory model; an arithmetic-logic unit (ALU); and a framebuffer-based pixel display. The CPU is minimal by design — every component maps directly to a concept taught in class.

# 2. Hardware Summary

| Component | Specification |
| --- | --- |
| Word size | 8 bits (unsigned) |
| Register file | 16 general-purpose registers: R0–R15 |
| Memory | 256 bytes of flat, byte-addressable RAM (addresses 0x00–0xFF) |
| Program Counter (PC) | 8-bit; holds byte address of next instruction to fetch |
| Instruction Register (IR) | 3 bytes wide — holds one fetched instruction (opcode, A, B) |
| Framebuffer / Screen | 64 × 64 monochrome pixel grid (W=64, H=64) |
| Instruction width | Fixed 3 bytes: [opcode][operand A][operand B] |
| Instruction count | 20 instructions (opcodes 0–19) |

# 3. Memory Model

Memory is a linear array of 256 bytes (indices 0–255). Both program code and data share the same address space — there is no separation between code and data memory (von Neumann architecture).

Programs are loaded starting at address 0. Because every instruction is exactly 3 bytes wide, instruction N begins at byte address N × 3. The PC always holds a byte address; when the CPU fetches an instruction it reads bytes at PC, PC+1, and PC+2, then advances PC by 3.

Jump targets in JMP and JNZ instructions are specified as instruction numbers (not byte addresses). The CPU internally converts: byte_address = instruction_number × 3.

# 4. Register File

There are 16 registers, numbered 0 through 15 (R0–R15). All registers are 8-bit and unsigned. There are no special-purpose registers — any register may be used as a source or destination for any instruction.

# 5. Instruction Set Architecture

## 5.1 Instruction Encoding

Every instruction is exactly 3 bytes:

| Byte 0 | Byte 1 | Byte 2 |
| --- | --- | --- |
| Opcode (0–19) | Operand A | Operand B |

Operands A and B are interpreted differently depending on the instruction (see Section 5.3). Unused operand bytes should be set to 0 by convention.

## 5.2 Opcode Table

Opcodes are grouped by function.

| Opcode | Mnemonic | A | B | Description |
| --- | --- | --- | --- | --- |
| Data Movement |  |  |  |  |
| 0 | LDI | Rd | imm8 | Load immediate: Rd ← imm8. |
| 1 | MOV | Rd | Rs | Copy register: Rd ← Rs. |
| 6 | LD | Rd | Ra | Load from memory: Rd ← mem[Ra]. |
| 7 | ST | Ra | Rs | Store to memory: mem[Ra] ← Rs. |
| Arithmetic |  |  |  |  |
| 2 | ADD | Rd | Rs | Add: Rd ← Rd + Rs. Wraps on overflow (mod 256). |
| 3 | SUB | Rd | Rs | Subtract: Rd ← Rd − Rs. Wraps on underflow (mod 256). |
| Bitwise / Logic |  |  |  |  |
| 4 | AND | Rd | Rs | Bitwise AND: Rd ← Rd & Rs. |
| 5 | OR | Rd | Rs | Bitwise OR: Rd ← Rd ¦ Rs. |
| 13 | XOR | Rd | Rs | Bitwise XOR: Rd ← Rd ^ Rs. |
| 12 | NOT | Rd | --- | Bitwise NOT: Rd ← ~Rd. Operand B is ignored. |
| 14 | LSL | Rd | Rs | Logical shift left: Rd ← Rd << (Rs & 7). Vacated bits filled with 0. |
| 15 | LSR | Rd | Rs | Logical shift right: Rd ← Rd >> (Rs & 7). Vacated bits filled with 0. |
| Control Flow |  |  |  |  |
| 8 | JMP | target | --- | Unconditional jump: PC ← target × 3. Operand B is ignored. |
| 17 | JMPI | Ra | --- | Indirect jump: PC ← reg[Ra] × 3. Operand B is ignored. |
| 9 | JNZ | Rc | target | Jump if not zero: if Rc ≠ 0 then PC ← target × 3. Tests register value directly. |
| 16 | JZ | Rc | target | Jump if zero: if Rc = 0 then PC ← target × 3. Tests register value directly. |
| 11 | HALT | --- | --- | Stop execution. Both operand bytes are ignored. |
| Display |  |  |  |  |
| 10 | DRAW | Rx | Ry | Draw pixel on: screen[Ry][Rx] ← 1. Silently ignored if x ≥ 64 or y ≥ 64. |
| 18 | DRAWOFF | Rx | Ry | Draw pixel off: screen[Ry][Rx] ← 0. Silently ignored if x ≥ 64 or y ≥ 64. |
| 19 | CLEAR | mode | --- | Clear display: mode 0 = all pixels off; mode 1 = all pixels on. Operand B ignored. |

## 5.3 Operand Conventions

The following operand naming conventions are used throughout this document:

| Symbol | Meaning |
| --- | --- |
| Rd | Destination register (index 0–15) |
| Rs | Source register (index 0–15) |
| Ra | Address register — its value is used as a memory address |
| Rc | Condition register — tested for zero / nonzero |
| Rx / Ry | Register whose value is used as an X or Y pixel coordinate |
| imm8 | 8-bit immediate value (0–255), stored directly in operand byte B |
| target | Instruction number for jump destination (byte address = target × 3) |

# 6. ALU Behavior

The ALU supports eight operations: ADD, SUB, AND, OR, XOR, NOT, LSL, LSR. All arithmetic is performed on unsigned 8-bit values with natural wrap-around (modulo 256). Shift amounts are masked to the low 3 bits (range 0–7). There is no carry flag, overflow flag, or signed arithmetic mode.

# 7. Framebuffer & Display

The CPU includes a 64 × 64 monochrome framebuffer. Each pixel is either on (1) or off (0). The framebuffer is separate from the 256-byte memory — it cannot be read or written through LD/ST instructions; it is accessible only via DRAW, DRAWOFF, and CLEAR.

Coordinate origin (0, 0) is at the top-left corner. The X axis increases to the right; the Y axis increases downward. Pixel (x, y) maps to screen[y][x] in row-major order.

DRAW sets a pixel on. DRAWOFF sets a pixel off. CLEAR with mode 0 blanks the entire framebuffer; CLEAR with mode 1 fills it completely. Out-of-bounds DRAW and DRAWOFF instructions (x ≥ 64 or y ≥ 64) are silently ignored.

# 8. CPU Execution Model

The CPU follows a standard three-stage loop:

- Fetch: Read 3 bytes from memory at address PC, PC+1, PC+2 into the instruction register. Increment PC by 3.

- Decode: Interpret IR[0] as the opcode, IR[1] as operand A, and IR[2] as operand B.

- Execute: Carry out the operation, updating registers, memory, the framebuffer, or the PC as required.

Execution halts when a HALT instruction is decoded, or when PC would advance past address 253 (the last valid 3-byte instruction start in a 256-byte memory). An unrecognized opcode (any value outside 0–19) results in undefined behavior.

# 9. Reset State

On initialization (reset), all of the following are zeroed:

- All 16 registers (R0–R15 = 0)

- Program Counter (PC = 0)

- Halted flag (false)

- Memory (all 256 bytes = 0)

- Framebuffer (all 64×64 pixels = 0)

Note: when testing multiple programs sequentially, reset must be called between runs to avoid register, memory, or framebuffer contamination from previous programs.

# 10. Assembly Language Format

tinyCPU programs are written as flat byte arrays. Each instruction occupies exactly 3 consecutive bytes: opcode, operand A, operand B. Instruction addresses are implicit — instruction N lives at byte offset N × 3.

Example — load the value 42 into R0, then halt:

uint8_t prog[] = { LDI, 0, 42, // Instruction 0: R0 = 42 HALT, 0, 0, // Instruction 1: stop };

When writing loop bodies, count instruction numbers carefully — JMP and JNZ take instruction numbers, not byte offsets.

# 11. Worked Example — Loop with Counter

The following program counts R0 from 0 to 63 and halts. It demonstrates LDI, ADD, SUB, and JNZ.

// Instr# Bytes Meaning // 0 LDI, 0, 0 R0 = 0 (loop counter) // 1 LDI, 1, 1 R1 = 1 (increment) // 2 LDI, 2, 63 R2 = 63 (limit) // 3 ADD, 0, 1 R0 += R1 // 4 SUB, 2, 0 R2 -= R0 (becomes 0 when R0 reaches 63) // 5 JNZ, 2, 3 if R2 != 0 goto instr 3 // 6 HALT, 0, 0 uint8_t prog[] = { LDI, 0, 0, // R0 = 0 LDI, 1, 1, // R1 = 1 LDI, 2, 63, // R2 = 63 ADD, 0, 1, // R0 += R1 SUB, 2, 0, // R2 -= R0 JNZ, 2, 3, // if R2 != 0 goto instr 3 HALT, 0, 0, };